AI Project Brief — Automatic TCR / RQD Calculation from Borehole Core Box Images

Prepared for: AI Engineer / Trainee
Purpose: Build a production-ready computer vision pipeline that automatically estimates geotechnical core metrics from borehole core box images.

Note: Watch Video that explain core analysis. (if you have any questions do not hesitate)

# 1. Executive Summary

The goal of this project is to build an AI-assisted system that calculates Total Core Recovery (TCR) and Rock Quality Designation (RQD) from images of core boxes. Each core run is 1.5 m (150 cm). The system should process images containing multiple horizontal runs, identify recovered core pieces, assign them to the correct run, measure their lengths using the image scale / depth markers, and output per-run and total TCR and RQD values.

# 2. Core Definitions

TCR (%) = (sum of all recovered core lengths in run / 150 cm) × 100

RQD (%) = (sum of sound core pieces ≥ 10 cm in run / 150 cm) × 100

# 3. Recommended Annotation Classes

run_band — polygon or box covering each horizontal run

depth_marker — labels such as 0.00, 1.50, 3.00, 4.50, etc.

rqd_piece — sound recovered piece that should count toward RQD (≥ 10 cm)

non_rqd_piece — recovered piece that counts toward TCR but not toward RQD

# 4. Recommended Architecture

Detect or segment each horizontal run (run band).

Detect or segment each recovered core piece inside the run.

Classify each piece as either RQD-eligible or not RQD-eligible.

Use scale / depth markers / geometry to convert pixel lengths to real-world lengths.

Assign each detected piece to the correct 1.5 m run.

Compute TCR and RQD using deterministic formulas.

Expose results in a reviewable UI so engineers can correct mistakes.

# 5. Engineering Notes

Do not train one model to directly output TCR or RQD.

Use YOLO segmentation rather than only bounding boxes if possible.

Length should be computed from geometry, not learned as a class.

TCR includes all recovered pieces; RQD includes only sound pieces ≥ 10 cm.

Prefer a semi-automatic reviewed workflow in Milestone 1.

# 6. Deliverables Expected

Google Colab training notebook

Inference notebook or script

Trained weights

Post-processing script for TCR/RQD

Simple UI for upload / review / export

Documentation and limitations

# 7. Appendix

Insert representative labeled images, screenshots, and expected output tables here.

# 8. Data